Oct 31, 2013

Written examination exercises,

Course “Forest Hydrology” and “Forest and Hillslope Hydrology”

Problem 1

For a catchment 50 km2 wide you need to compute the runoff depth following a storm with duration of 3 hours and depth of 90.0 mm. The catchment is characterised by an overall CN equal to 60, Ia=0.05 Smax.

Compute

- the runoff depth in mm ;

- the runoff coefficient.                             .

Results:

1. runoff depth:	                                		mm

2. runoff coefficient:

Problem  2

Rainfall in a catchment 50 km2 wide is monitored by a network of 3 stations, for which the point rainfall during a flood is:

Station 1: 200 mm

Station 2: 200 mm

Station 3:  80 mm

Use the Thiessen method to compute the event basin averaged rainfall. The Thiessen weights are as follows:

Station 1: 0.25

Station 2: 0.25

Station 3: 0.50

The catchment is characterised by an overall CN equal to 60, Ia=0.04 Smax.

Compute

- the runoff depth in mm ;

- the runoff depth in m3;

- the runoff coefficient.                             .

Answers

Thiessen

Areal mean precipitation:        		mm

Runoff depth:				mm

Runoff ratio:				(-)

Problem  3

Repeat the computation of Problem 2 by considering the stations equally representative, and compute the corresponding runoff depth and runoff coefficient. Discuss the results you have obtained.

Discussion on the differences

Answers

Arithmetic average

Areal mean precipitation:        		mm

Runoff depth:				mm

Runoff ratio:				(-)